TED UNIVERSITY

CMPE 491 / SENG 491 Senior Project

Center of Gravity Detection, Information, and Balancing System in Aircraft

Project Proposal

10.10.2025

Team Members:

Sude İpekci, Computer Engineering

Yasin Anber, Computer Engineering

Selin Göç,  Computer Engineering

Elif Sude Memiş, Software Engineering

Supervisor:

Prof. Dr. Hakkı Gökhan İLK

Jury Members:

Dr. Ali BERKOL

Dr. Ayşe Yasemin SEYDİM

Project Name: Center of Gravity Detection, Information, and Balancing System in Aircraft

Project Webiste Link: http://codenge.vercel.app

This project aims to design and prototype a real-time Center of Gravity (CoG) detection, visualization, and automatic balancing system for small-scale aerial platforms. tThe precise location of the center of gravity is crucial for flight safety, stability, maneuverability, and fuel efficiency throughout aircraft design and operation. Performance might be adversely affected and unsafe flight situations can result from even little deviations brought on by passenger loading, cargo placement, or fuel usage. A small-scale helicopter platform is set up on load-cell sensors that are positioned below each landing gear point in this investigation. These sensors detect the distributed load continually, and the instantaneous CoG coordinates are computed by an embedded system based on Arduino. After that, the measured and ideal CoG positions are shown in real time, and any variations are tracked to guarantee safe bounds.

To actively correct CoG shifts, the system includes an automatic balancing mechanism using servo motors and a lead-screw-driven movable mass. When a shift in CoG is detected, the embedded algorithm commands the servo system to reposition a counterweight along the X-Y plane, restoring the CoG to the predefined optimal location. This concept mimics real aerospace weight-trim systems and provides a scalable foundation for future autonomous aircraft weight management technologies. The proposed system lays the groundwork for more advanced real-time load-balancing solutions in UAVs, helicopters, and other aerial platforms—enabling smarter, safer, and more efficient flight systems.

This project is developed within the scope of the TUSAŞ Lift-Up Program.