TED UNIVERSITY

CMPE 491 / SENG 491 Senior Project

Center of Gravity Detection, Information, and Balancing System in Aircraft

Project Specifications Report

7.11.2025

Team Members:

Yasin Anber, Computer Engineering

Sude İpekci, Computer Engineering

Selin Göç,  Computer Engineering

Elif Sude Memiş, Software Engineering

Supervisor:

Prof. Dr. Hakkı Gökhan İLK

Jury Members:

Dr. Ali BERKOL

Dr. Ayşe Yasemin SEYDİM

Table of Contents

1   Introduction	3

1.1	Project Description	3

1.1.1	Overview	3

1.1.2	Objectives	3

1.1.3	Constraints	4

1.2	Professional and Ethical Issues	4

2	Requirements	5

2.1	Functional Requirements:	5

2.2	Non-Functional Requirements:	5

3	References	6

# 1   Introduction

This report provides the specifications for the project titled, "Center of Gravity Detection, Information, and Balancing System in Aircraft". The main objective of this project is to develop a software system that interprets real-time analog data obtained from load cells and determines the optimal center of gravity using an algorithm. In addition, when a deviation from the optimal center of gravity is detected, the system aims to restore the aircraft’s center of gravity to its ideal position by moving the balancing weight to the optimal coordinate with the help of servo motors. In line with this purpose, the project involves the development, programing, design, and implementation of a system that calculates the center of gravity and continuously maintains it at the optimal point. To create a dependable and effective CG balancing system that improves flight safety, performance, and fuel efficiency, we will move through system design, hardware and software integration, and testing phases throughout the project. Lastly, as part of this project, a website was developed that demonstrates the system's capabilities and design in a more interactive and accessible way. The website can be accessed at [https://codenge.vercel.app/].

## Project Description

### Overview

The Automatic Center of Gravity (CG) Detection and Balancing System aims to automatically calculate the aircraft’s center of gravity and, when necessary, use a recalculation algorithm to maintain it at the optimal point [5][6]. Through its design and software, the system is intended to enhance the aircraft’s stability and safety [6].

The scope of our project will include:

• Strain gauge type load cells located under each wheel of the aircraft.

• A platform for measuring weight distribution in real time using load cells.

• An Arduino-based embedded system and HX711 amplifier for gathering and processing data.

• A balancing mechanism powered by a servo that can move precisely along the X and Y axes [6].

• Software programs that automatically determine CG position and regulate servo motors movements.

• Two lead screw mechanisms and servo motors are used together to enable precise movement of the balancing weight along two axes (X and Y).

• A display interface that shows system status and CG data.

• Modules for testing and calibration to guarantee response dependability and measurement accuracy.

### Objectives

| Objective | Brief Description of the Objective |
| --- | --- |
| Build a system that automatically detects and balances the center of gravity (CG). | Develop a system that can automatically modify a balancing mechanism to maintain stability by detecting the aircraft’s center of gravity in real time.. |
| Ensure the weight measurement is correct and in real time.. | To provide accurate, continuous data on the distribution of weight across the aircraft platform, strain gauge type load cells are used together with the HX711 module, which functions as both an analog-to-digital converter (ADC) and a signal amplifier.. |
| Improve safety and flying stability. | Maintain an optimal center of gravity to prevent instability, enhance maneuverability, and reduce flight risks.. |

### Constraints

- Economic Constraints: The cost for the prototype must be low, and its components must be accessible by using relatively easier to find components (e.g., load cells, HX711, Arduino, servo motors, lead screw). Fundamentally, the system should show that it is just a low-cost prototype that is used for educational purposes.

- Environmental Constraints: To achieve minimum electronic waste and environmental impact, electronic components must be selected in consideration of recyclability and low energy consumption.

- Social and Political Constraints: There will be no dual-use or military applications on this project. It emphasizes research, education, and civil aviation while favorably influencing technological advancement without raising moral questions.

- Ethical Constraints: The IEEE [2] and ACM Codes [1] of Ethics will be applied to all design and testing processes to respect intellectual property and provide honesty and fairness. The system and the data received from it will be used only for academic and research purposes [3].

- Health and Safety Constraints: The experimental model will function within safe mechanical limits and voltage rates. In order to prevent injury, moving parts of the system such as servos and lead screws will be secured. There will be no high-power systems or hazardous materials used.

- Manufacturability Constraints: The system will be developed by standard manufacturing methods to provide ease of replication. In addition, the components are modular, enabling scalability in further iterations.

- Sustainability Constraints: The system prioritizes long-term usability and ease of maintenance. Therefore, some components such as motors and sensors can be replaced separately, aiming to reduce waste. In addition to that, it is possible to deploy software updates without changing to hardware.

## Professional and Ethical Issues

This project aligns with professional responsibilities defined by ACM and IEEE Codes of Ethics [1][2]. Honesty and transparency are maintained through accurate documentation of all design and results [2]. All project phases, reports, and software documentation are stored in a shared folder accessible to all group members and are automatically backed up monthly. Public welfare and safety are prioritized by developing a system that enhances aircraft stability and reliability. Materials and processes are chosen for sustainability, while all intellectual property and external references are properly credited [4]. The development team ensures technical competence by adhering to engineering standards in hardware interfacing, signal processing, and embedded control [3][5].

Ethically, the system is developed for civil and educational purposes, avoiding any harmful or military applications [4]. The design also considers environmental sustainability, minimizing electronic waste and power consumption during testing and operation.

# Requirements

## Functional Requirements:

- Weight Measurement: The system shall receive weight data from the load cells and convert them into digital form using the HX711 module.

- Center of Gravity Calculation: The system shall calculate the CG coordinates according to the input from multiple loadcell sensors in real time.

- Automatic Balancing: To maintain equilibrium, the system shall control servo motors so that they move a balancing weight along X and Y axes.

- Data Visualization: To provide real-time feedback to the user, the system shall display important information including the current total weight, individual load readings, and the calculated CG position on a graphical interface such as a computer dashboard or serial plotter.

- Reference Comparison: The system shall store a preset optimal CG position and continuously compare it to the current CG. Then, if the deviation rises above a predetermined threshold, the system shall start corrective balancing.

- System Calibration: Before operating, the system shall undergo a preliminary calibration procedure to zero-load cells and create precise baseline measurements.

## Non-Functional Requirements:

- Performance: The system shall process and update CG data every 2 seconds or less to ensure responsive real-time balancing.

- Accuracy: The system shall detect and correct CG deviations within ±2% of the actual balance point, preserving stable equilibrium under normal operating conditions.

- Reliability: The system shall continue to function consistently under variable load conditions without data loss.

- Safety: The system shall run at a voltage lower than 12V DC, and all moving parts must have mechanical limits to prevent damage or injury. In this prototype, the operating voltage is limited to 5V DC to ensure safe and stable operation of the Arduino, HX711 module, and servo motors.

- Usability: The graphical interface shall provide clear and user-friendly visual indicators for sensor data, balance status, and system alerts.

- Maintainability: The system shall have modular software and hardware components so that individual parts can be replaced or updated without requiring a complete system redesign.

- Scalability: The design shall facilitate integration with autonomous flight control systems or expansion to larger aircraft platforms in later versions.

- Environmental Robustness: The system shall operate reliably under indoor laboratory conditions (temperature between 15–35°C and humidity ≤70%), minimizing drift caused by ambient environmental changes.

# References

[1] Association for Computing Machinery. (2018). ACM Code of Ethics and Professional Conduct.https://www.acm.org/code-of-ethics

[2] IEEE. (2020). IEEE Code of Ethics. IEEE. https://www.ieee.org/about/corporate/governance/p7-8

[3] IEEE Computer Society & ACM. (1999). Software Engineering Code of Ethics and Professional Practice. IEEE-CS/ACM Joint Task Force. https://www.computer.org/education/code-of-ethics

[4] Floridi, L. (2023). Computer and information ethics. In E. N. Zalta & U. Nodelman (Eds.), The Stanford Encyclopedia of Philosophy (Fall 2023 Edition). Stanford University. https://plato.stanford.edu/archives/fall2017/entries/ethics-computer/

[5] Pandit, J. K., Kumbar, P. B., Kumar, S. S., Thyagaraj, M. R., & Govinda, K. V. (2015). Accurate estimation of Center of Gravity of spacecraft electronic packages – enhancing their durability. Journal of Materials Science & SurfaceEngineering,3(1),185-189. https://www.academia.edu/98629454/Accurate_Estimation_of_Center_of_Gravity_of_Spacecraft_Electronic_Packages_Enhancing_their_Durability

[6] Zhao, X., Yuan, Y., Dong, Y., & Zhao, R. (2021). Optimization approach to the aircraft weight and balance problem with the centre of gravity envelope constraints. IET Intelligent Transport Systems, 15(10), 1269–1286. https://ietresearch.onlinelibrary.wiley.com/doi/10.1049/itr2.12096